RL Reward Function - Updated TD3D Hopper

This document converts the original RL_RewardFunction.pdf draft into an updated Word document and reflects the current reward/state-machine implementation in hopper_env.py.

State Machine

Phases: climb -> flip -> recovery -> hover -> done

The environment starts in climb. Phase transition bonuses are one-shot rewards:

climb -> flip: +50 when 9.0 <= z <= 10.0 and upright_score > 0.85

flip -> recovery: +150 when flip_progress > 0.95 and upright_score > 0.85

recovery -> hover: +100 when flip_progress > 0.95 and upright_score > 0.95

hover success: +2500 after stable hover_timer >= 5.0 seconds

Any fail: -1000 and done=True

Shared Metrics

z: bottom point height of the rocket, not COM height.

relative_pos: bottom_point - target_pos.

rel_dist: norm(relative_pos[:2]).

v: norm(linear_vel).

w: norm(angular_vel).

joint_speed: norm([tvc_yaw_velocity, tvc_pitch_velocity]).

upright_score: dot(body_axis, world_z).

flip_progress: accumulated positive pitch/somersault progress around world Y axis divided by 2*pi.

positive_flip_axis_rate: max(dot(angular_vel, [0,1,0]), 0). Only this direction increases flip progress.

off_axis_angular_speed: angular velocity outside the desired flip axis.

expected_axis_alignment: alignment between current body axis and expected body axis along the desired flip path.

Climb Reward

target_z = 10.0

if z < 10.5: reward -= abs(z - target_z) * 1.5

else: reward -= 0.5 * z

low_altitude_error = max(9.0 - z, 0.0)

reward -= low_altitude_error * 1.0

reward -= low_altitude_error ** 2 * 0.5

reward -= abs(relative_pos[0]) * 1.15

reward -= abs(relative_pos[1]) * 1.25

reward -= rel_dist ** 2 * 1.5

reward -= w * 2.0

reward -= joint_speed * 0.09

reward += upright_score * 15.0

reward -= (horizontal_speed + vertical_speed) * 3.4

Fail: abs(relative_pos[0]) > 3.0 or abs(relative_pos[1]) > 2.0

Flip Reward

flip_height_error = abs(z - 10.0)

reward -= flip_height_error * 0.35

progress_delta = abs(flip_progress) - abs(last_step_flip_progress)

reward += progress_delta * 35.0

if progress_delta <= 0.001: reward -= 0.35

if flip_progress > 1.0: reward -= flip_progress * 1.0

if z < 5.0: reward -= z * 2.5

if abs(upright_score - last_upright_score) > 0.3: reward -= upright_change * 30.0

if w > 30.0: reward -= w * 1.0

reward += expected_axis_alignment * 8.0

reward -= off_axis_angular_speed * 0.65

reward -= joint_speed * 0.12

reward -= side_deviation * 3.0

reward -= rel_dist * 3.0

reward -= rel_dist ** 2 * 1.5

if flip_progress > 0.75: reward -= w * 0.45

reward -= 0.35 * abs(spin_about_body_axis)

Fail: z < 3.5 and flip_progress < 0.15, or w > 70.0

Recovery Reward

reward -= v * 1.5

reward -= w * 1.45

reward -= joint_speed * 0.09

reward += 30.0 * upright_score

reward += 30.0 * flip_progress

reward -= rel_dist * 30.0

reward += (last_rel_dist - rel_dist) * 50.0

Fail: v > 70.0 or w > 70.0 or rel_dist > 1.5

Hover Reward

reward -= v * 1.5

reward -= w * 1.45

reward -= joint_speed * 0.09

reward += 30.0 * upright_score

reward -= rel_dist * 30.0

Fail: rel_dist > 1.5 or v > 70.0 or w > 70.0

Success: hover_timer >= 5.0 seconds, reward += 2500.0

Global Fail Conditions

Bad physics: non-finite qpos/qvel/qacc, linear qvel > 120, angular qvel > 180, z < 0.05, or z > 18.0.

Global XY escape: abs(relative_pos[0]) > 6.0 or abs(relative_pos[1]) > 6.0.

Global speed limit: v > 45.0 or w > 60.0.

Global height limit: z < 0.05 or z > 18.0.

Physical Model Updates

Max thrust defaults to 3.6 kgf = 3.6 * 9.80665 N.

Max TVC angle defaults to 20 degrees.

TVC servo speed is limited to 60 degrees / 0.14 seconds = 428.57 deg/s.

Viewer camera follows the rocket body while watching or visualizing.